Enhanced Magnetization in Metastable Disordered Fe4Ga

Paul W. White1, Connor McCollar1, Jacob Peterson2, Manish Neupane3, Mohd Anas3, Parashu Kharel3, Tula R. Paudel1

1 Physics Department, South Dakota School of Mines and Technology

2 Engineering and Mining Station, South Dakota School of Mines and Technology

3 Department of Chemistry, Biochemistry and Physics Department, South Dakota State University

Recent research has focused on developing soft magnetic materials with superior magnetic properties for next-generation inductor cores in electrified vehicles. We explored an alternative alloy to the traditional Fe-Nb-Cu-Si-B alloy using Ga substitution for Si. The novel Fe-Nb-Cu-Ga-B alloys were found to have a ~35% larger magnetization from VSM measurements. Structural characterization performed with XRD and SAED determined the crystal structure and microstructure. The diffraction patterns show a competition between the stable ordered Fe3Ga (sp. gr. 225) phase and metastable disordered Fe4Ga (sp. gr. 229) phase. From the complex microstructure, we were able to determine that the Fe4Ga phase formed as an intermediate step as the Fe3Ga phase was crystallizing due to the rapid cooling rate ~106C/s. First principles calculations found that the Fe magnetic moment increased by 35% in the Fe4Ga phase compared to the Fe3Ga phase, correlating with the experimentally measured increase in magnetization. Our findings show a significant increase in magnetization due to the presence of metastable disordered nanocrystalline phases, which may be the future of soft magnetic materials.